Perfect PAMSY 🙌 — let’s rewrite your explanation clearly and correctly:

### ✅ Corrected Answer

When PC1 pings PC3:

- Step 1 — ARP Request (Broadcast)

  - PC1 needs PC3’s MAC address but doesn’t know it yet.

  - PC1 sends an ARP request: “Who has IP 192.168.1.3? Tell me your MAC.”

  - This ARP request is sent as a broadcast frame with destination MAC = FF:FF:FF:FF:FF:FF.

  - All devices in the LAN (PC2, PC3, PC4, SW1, SW2) receive the broadcast.

- Step 2 — ARP Reply (Unicast)

  - Only PC3 recognizes its own IP in the request.

  - PC3 replies with its MAC address.

  - This reply is a unicast frame sent directly to PC1 (destination = PC1’s MAC).

  - Other PCs (PC2, PC4) do not process this reply.

- Step 3 — Ping (ICMP Echo Request/Reply)

  - Now PC1 knows PC3’s MAC.

  - PC1 sends the ping (ICMP Echo Request) as a unicast frame to PC3.

  - PC3 responds with an ICMP Echo Reply, also unicast back to PC1.

- Step 4 — Switch MAC Table Learning

  - As frames travel through SW1 and SW2, each switch learns the source MAC addresses and stores them in its MAC address table.

  - This way, future traffic is forwarded directly without broadcasting.

### 📝 Key Points

- Broadcast (ARP Request) → received by everyone, but only PC3 replies.

- Unicast (ARP Reply + Ping) → exchanged only between PC1 and PC3.

- Switches learn MACs dynamically during this process.

-   The ARP request/reply happens at Layer 2 (Data Link) because it deals with MAC addresses.

-   The ICMP echo request/reply happens at Layer 3 (Network) because it uses IP addresses.

-   Both are encapsulated into Ethernet frames at Layer 2 when sent across the physical network.

-   ARP = Layer 2 (Data Link)

-   ICMP Echo Request/Reply = Layer 3 (Network / Internet Layer in TCP/IP)