# DHCP (Dynamic Host Configuration Protocol)

Study notes — organized and cleaned up for review.

# 1. Overview

DHCP allows a host to automatically and dynamically learn network configuration details — such as IP address, subnet mask, default gateway, and DNS server — without manual/static configuration.

# 2. What DHCP Does

- DHCP acts like a helper that automatically gives each computer or phone its IP address.

- Without DHCP, you would have to manually type the IP address, subnet mask, gateway, and DNS server on every device.

- It is an essential part of modern networks — when a phone or laptop connects to Wi-Fi, it automatically receives network access without the user specifying any addressing details.

In short: the host says “I need an IP!” and the DHCP server (or router acting as one) replies “Here you go — use this IP, gateway, and DNS.”

## 2.1 Where DHCP Is Used

- DHCP is typically used for client devices such as workstations (PCs), phones, etc.

- Devices such as routers and servers are usually configured manually (static IP), since they need a fixed address to reliably perform their functions — for example, if a default gateway's address kept changing, it would break connectivity for everyone relying on it.

## 2.2 Who Runs the DHCP Server

- Small network (e.g. home network): the router typically acts as the DHCP server for hosts on the LAN.

- Large network (e.g. enterprise): the DHCP server is usually a dedicated Windows or Linux server.

# 3. The DORA Process

DORA describes the four-message exchange used by a client to obtain an IP address from a DHCP server.

- Discover — Client broadcasts a request for an IP address.

- Offer — Server proposes an IP address and configuration.

- Request — Client accepts the offered IP address.

- Acknowledge (ACK) — Server confirms and finalizes the lease.

## 3.1 Discover

The client sends a broadcast message asking if there is a DHCP server on the local network, since it needs an address (“Is there a DHCP server out there? I need an IP address!”).

## 3.2 Offer

After receiving a Discover broadcast, the DHCP server replies with a DHCP Offer containing a proposed IP address plus other settings (subnet mask, default gateway, DNS server, lease time).

## 3.3 Request

After receiving one or more Offers, the client picks one and sends a broadcast DHCP Request stating which offer it accepts. This also implicitly tells other DHCP servers that their offers were not taken.

In practice, broadcast is commonly used for safety, though unicast is technically allowed if the server can reach the client directly.

## 3.4 Acknowledge (ACK)

The server confirms and finalizes the lease, giving the client full configuration details. The client can then communicate on the network.

## 3.5 DORA Summary Table

| Step | Direction | Message Type |
| --- | --- | --- |
| DISCOVER | Client → Server | Broadcast |
| OFFER | Server → Client | Unicast or Broadcast |
| REQUEST | Client → Server | Broadcast |
| ACK | Server → Client | Broadcast or Unicast |

# 4. The Role of the MAC Address

- Without an IP address, a PC cannot send unicast frames — it must start with broadcast.

- The MAC address is what allows the server to reply specifically to that PC: the Discover message contains the client's MAC address, so the server (and only that client) can recognize the matching Offer/ACK.

- Even though the Offer may be broadcast, only the client with the matching MAC address accepts it.

Key point: the MAC address is the “ID card” that ensures the right host gets the right IP.

# 5. Static vs Dynamic Addressing

- Static/manual IPs — used for critical devices (routers, servers) for stability and predictability.

- Dynamic IPs (DHCP) — used for normal devices (PCs, phones, printers) for convenience.

# 6. DHCP Relay Agents

## 6.1 Why Broadcasts Don't Reach a Centralized Server

- DHCP Discover is a broadcast message (destination IP 255.255.255.255, destination MAC FF:FF:FF:FF:FF:FF).

- Broadcasts are limited to the local subnet — they do not cross routers.

- If the DHCP server is centralized in a different subnet, it will never directly hear a client's Discover message.

## 6.2 How Enterprises Solve This

Large enterprises typically use a centralized DHCP server (often Windows or Linux) rather than one per subnet. To make this work across subnets, they configure a DHCP Relay Agent, usually on a router or Layer 3 switch interface, for each subnet:

- The relay agent listens for DHCP broadcasts on its local subnet.

- When it hears one, it forwards the message as unicast to the centralized DHCP server's IP address.

- The server replies to the relay agent, which then delivers the response back to the client.

## 6.3 Placement: One Relay Agent Per Subnet

- You do not need a single relay agent for the whole enterprise.

- Instead, configure one relay agent per subnet, typically on the router interface connecting that subnet.

- Example: 5 subnets → 5 relay agents, each forwarding to the same centralized DHCP server.

## 6.4 Small vs Large Networks

- Small network (single subnet): no relay agent needed. The DHCP server sits in the same subnet as clients and hears broadcast Discover messages directly.

- Large / multi-subnet network: relay agents are required on each subnet so client broadcasts can reach the centralized DHCP server.

# 7. Cisco IOS Configuration

Note: the original note lists this as a topic but does not yet contain the configuration steps/commands. Add them here once available (e.g. ip dhcp pool, ip dhcp excluded-address, ip helper-address for relay).

# 8. Key Takeaways

- DHCP = automatic IP assignment for client devices.

- DORA = Discover → Offer → Request → ACK.

- MAC address = the identifier ensuring the right host receives the right IP.

- Small networks: local DHCP server, no relay needed.

- Large networks: centralized DHCP server + one relay agent per subnet (commonly configured with ip helper-address on Cisco routers).