DHCP (Dynamic Host Configuration Protocol)

CCNA Study Notes

Organized, cleaned up, and expanded for review.

# 1. Overview

DHCP allows a host to automatically and dynamically learn network configuration details — such as IP address, subnet mask, default gateway, and DNS server — without manual/static configuration.

# 2. What DHCP Does

- DHCP acts like a helper that automatically gives each computer or phone its IP address.

- Without DHCP, you would have to manually type the IP address, subnet mask, gateway, and DNS server on every device.

- It is an essential part of modern networks — when a phone or laptop connects to Wi-Fi, it automatically receives network access without the user specifying any addressing details.

In short: the host says “I need an IP!” and the DHCP server (or router acting as one) replies “Here you go — use this IP, gateway, and DNS.”

## 2.1 Where DHCP Is Used

- DHCP is typically used for client devices such as workstations (PCs), phones, etc.

- Devices such as routers and servers are usually configured manually (static IP), since they need a fixed address to reliably perform their functions — if a default gateway's address kept changing, it would break connectivity for everyone relying on it.

## 2.2 Who Runs the DHCP Server

- Small network (e.g. home network): the router typically acts as the DHCP server for hosts on the LAN.

- Large network (e.g. enterprise): the DHCP server is usually a dedicated Windows or Linux server.

# 3. The DORA Process

DORA describes the four-message exchange used by a client to obtain an IP address from a DHCP server.

- Discover — Client broadcasts a request for an IP address.

- Offer — Server proposes an IP address and configuration.

- Request — Client accepts the offered IP address.

- Acknowledge (ACK) — Server confirms and finalizes the lease.

## 3.1 Discover

The client sends a broadcast message asking if there is a DHCP server on the local network, since it needs an address (“Is there a DHCP server out there? I need an IP address!”).

## 3.2 Offer

After receiving a Discover broadcast, the DHCP server replies with a DHCP Offer containing a proposed IP address plus other settings (subnet mask, default gateway, DNS server, lease time).

## 3.3 Request

After receiving one or more Offers, the client picks one and sends a broadcast DHCP Request stating which offer it accepts. This also implicitly tells other DHCP servers that their offers were not taken.

In practice, broadcast is commonly used for safety, though unicast is technically allowed if the server can reach the client directly.

## 3.4 Acknowledge (ACK)

The server confirms and finalizes the lease, giving the client full configuration details. The client can then communicate on the network.

## 3.5 DORA Summary Table

| Step | Direction | Message Type |
| --- | --- | --- |
| DISCOVER | Client → Server | Broadcast |
| OFFER | Server → Client | Unicast or Broadcast |
| REQUEST | Client → Server | Broadcast |
| ACK | Server → Client | Broadcast or Unicast |

# 4. The Role of the MAC Address

- Without an IP address, a PC cannot send unicast frames — it must start with broadcast.

- The MAC address is what allows the server to reply specifically to that PC: the Discover message contains the client's MAC address, so the server (and only that client) can recognize the matching Offer/ACK.

- Even though the Offer may be broadcast, only the client with the matching MAC address accepts it.

Key point: the MAC address is the “ID card” that ensures the right host gets the right IP.

# 5. Static vs Dynamic Addressing

- Static/manual IPs — used for critical devices (routers, servers) for stability and predictability.

- Dynamic IPs (DHCP) — used for normal devices (PCs, phones, printers) for convenience.

# 6. DHCP Relay Agents

## 6.1 Why Broadcasts Don't Reach a Centralized Server

- DHCP Discover is a broadcast message (destination IP 255.255.255.255, destination MAC FF:FF:FF:FF:FF:FF).

- Broadcasts are limited to the local subnet — they do not cross routers.

- If the DHCP server is centralized in a different subnet, it will never directly hear a client's Discover message.

## 6.2 How Enterprises Solve This

Large enterprises typically use a centralized DHCP server (often Windows or Linux) rather than one per subnet. To make this work across subnets, they configure a DHCP Relay Agent, usually on a router or Layer 3 switch interface, for each subnet:

- The relay agent listens for DHCP broadcasts on its local subnet.

- When it hears one, it forwards the message as unicast to the centralized DHCP server's IP address.

- The server replies to the relay agent, which then delivers the response back to the client.

## 6.3 Placement: One Relay Agent Per Subnet

- You do not need a single relay agent for the whole enterprise.

- Instead, configure one relay agent per subnet, typically on the router interface connecting that subnet.

- Example: 5 subnets → 5 relay agents, each forwarding to the same centralized DHCP server.

## 6.4 Small vs Large Networks

- Small network (single subnet) — no relay agent needed. The DHCP server sits in the same subnet as clients and hears broadcast Discover messages directly.

- Large / multi-subnet network — relay agents are required on each subnet so client broadcasts can reach the centralized DHCP server.

# 7. Cisco IOS Configuration

This section covers the actual configuration commands: setting up a DHCP pool, excluding addresses, running multiple pools, verifying leases, configuring a relay agent, and making a router itself a DHCP client.

## 7.1 Excluding Addresses from the Pool

ip dhcp excluded-address 192.168.1.1 192.168.1.10

This excludes the entire range from .1 to .10 — not just the endpoints. All addresses in between are reserved and will not be handed out automatically.

- Excluded addresses are usually reserved for devices that need a fixed IP: routers, servers, printers, access points.

- DHCP will then only assign addresses starting from the first address after the excluded range.

Excluded does not mean “forbidden to use.” It means “DHCP must not automatically assign it.” An excluded address can still be configured manually (statically) on a device — for example, the router itself typically uses .1 statically, which is exactly why it's excluded.

## 7.2 Creating a DHCP Pool

ip dhcp pool LAN

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 8.8.8.8

domain-name jeremysitlab.com

A DHCP pool is a block of addresses (plus related settings) that the router can hand out to clients. Each command inside the pool sets one piece of that configuration:

- network — tells the pool which subnet/range it is responsible for. network 192.168.1.0 /24 means the pool serves 192.168.1.0/24 — it does not mean the router hands out .0 itself; .0 is the network address and .255 is the broadcast address, neither of which is ever assigned to a client.

- excluded-address — addresses in this subnet that DHCP must leave alone (configured outside the pool, applies to the whole router).

- default-router — the gateway address given to clients.

- dns-server — the DNS server address given to clients.

- domain-name — the DNS suffix given to clients, e.g. jeremysitlab.com, so PC1 resolves as pc1.jeremysitlab.com.

## 7.3 How the Pool Decides What to Hand Out

The pool's usable range comes from the network command; the excluded-address command then removes part of that range:

| Address / Range | Role | Assignment |
| --- | --- | --- |
| 192.168.1.0 | Network address | n/a |
| 192.168.1.1 – .10 | Router, servers, printers | Excluded — static |
| 192.168.1.11 – .254 | Client PCs, phones, etc. | DHCP pool |
| 192.168.1.255 | Broadcast address | n/a |

When a PC requests an address, the router walks the pool and hands out the first available (non-excluded, non-leased) address — e.g. PC1 gets .11, PC2 gets .12, and so on.

## 7.4 Multiple DHCP Pools

A router needs one DHCP pool per network/subnet it serves. This is common wherever a router serves multiple VLANs — one pool cannot cover two different subnets.

! Pool for Employees (VLAN 10)

ip dhcp pool EMPLOYEES

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

! Pool for Students (VLAN 20)

ip dhcp pool STUDENTS

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

Rule of thumb: one network → one DHCP pool.

## 7.5 Verifying Leases

show ip dhcp binding

Shows every DHCP client currently assigned an IP address by this router — useful for confirming the pool is actually leasing addresses as expected.

## 7.6 Configuring a DHCP Relay Agent

A relay agent needs two things working together: a way to reach the DHCP server, and the command that actually forwards DHCP requests to it.

- Reachability — R1 needs a route to the DHCP server. If there isn't one already, configure a static route, or use a dynamic routing protocol such as OSPF or RIP.

- Forwarding — apply the helper address on the interface facing the clients:

interface <client-facing-interface>

ip helper-address <DHCP-server-IP>

A route and ip helper-address are not interchangeable — a route finds the road to the server, ip helper-address is what actually forwards the DHCP request along it. Both are required.

## 7.7 Router as a DHCP Client

A Cisco router's own interface can use DHCP to learn its IP address, the same way a PC does — it isn't limited to being a DHCP server or relay.

interface <interface-name>

ip address dhcp

With this configured, the interface sends a DHCP Discover and obtains its address dynamically, rather than being set with a static ip address command.

## 7.8 Command Quick Reference

| Command | Purpose |
| --- | --- |
| ip dhcp excluded-address <start> <end> | Reserve a range so DHCP never hands it out automatically |
| ip dhcp pool <name> | Create/enter a DHCP pool |
| network <addr> <mask> | Define which subnet this pool serves |
| default-router <addr> | Gateway address given to clients |
| dns-server <addr> | DNS server given to clients |
| domain-name <name> | DNS suffix given to clients |
| show ip dhcp binding | List clients currently leased an address |
| ip helper-address <server-ip> | Turn an interface into a DHCP relay agent |
| ip address dhcp | Have the router itself act as a DHCP client on an interface |

# 8. Key Takeaways

- DHCP = automatic IP assignment for client devices.

- DORA = Discover → Offer → Request → ACK.

- MAC address = the identifier ensuring the right host receives the right IP.

- Small networks: local DHCP server, no relay needed.

- Large networks: centralized DHCP server + one relay agent per subnet (ip helper-address on Cisco routers).

- One DHCP pool per subnet/VLAN; network sets the range, excluded-address reserves part of it for static devices.

- show ip dhcp binding verifies which clients currently hold a lease.

- A router can be a DHCP server, a DHCP relay agent, and/or a DHCP client — three separate, combinable roles.