NAT (Network Address Translation)

CCNA Study Notes

Organized and cleaned up for review.

# 1. Overview

NAT (Network Address Translation) is used to translate the source and/or destination IP address of a packet to a different IP address. This document covers:

- Private IP addresses — addresses you're free to use in your internal network, which don't have to be globally unique.

- An introduction to NAT and its purpose.

- The one type of NAT you need to know for the CCNA — static NAT — and how to configure it.

# 2. The IPv4 Address Shortage

IPv4 doesn't provide enough addresses for every device that needs one in the modern world.

- The long-term solution is to switch to IPv6.

- But migrating the entire world from IPv4 to IPv6 isn't simple, so three short-term solutions exist that let us keep using IPv4 for longer:

  - CIDR (Classless Inter-Domain Routing) — lets us forget about rigid IPv4 classes like /8, /16, /24.

  - Private IPv4 addresses

  - NAT

## 2.1 Private IPv4 Address Ranges

RFC 1918 (Request for Comments) specifies the following IPv4 ranges as private:

| Class | Range | CIDR Block |
| --- | --- | --- |
| Class A | 10.0.0.0 – 10.255.255.255 | 10.0.0.0/8 |
| Class B | 172.16.0.0 – 172.31.255.255 | 172.16.0.0/12 |
| Class C | 192.168.0.0 – 192.168.255.255 | 192.168.0.0/16 |

A natural question: why is the Class B range /12 instead of /16, and the Class C range /16 instead of /24? Because these are simply address ranges you're free to divide up however you want — don't get too attached to the old concept of address classes( what they mean is that the old idea of fixed address classes (Class A = /8, Class B = /16, Class C = /24) is outdated, because modern networking uses CIDR notation, which allows flexible prefix lengths like /12, /20, or /28 depending on actual needs. So instead of being locked into rigid “Class B” or “Class C” boundaries, you can freely divide IP ranges into whatever subnet sizes make sense, and that’s why they say don’t get too attached to the old concept of address classes.) . In modern networking, everyone uses CIDR notation, so address classes are largely a thing of the past.

These private addresses are free to use in your own network — they don't have to be globally unique.

# 3. What NAT Does

NAT (Network Address Translation) modifies the source and/or destination IP address of a packet.

- There are various reasons to use NAT, but the most common is to allow hosts with private IP addresses to communicate with hosts over the internet.

- There are several types of NAT, but for the CCNA the important one is source NAT and how to configure it on a Cisco router.

## 3.1 How Source NAT Works

- When a PC in a local network (PC1) wants to reach a remote server, it sends a packet using its own private IP address, addressed to the server (e.g. 8.8.8.8).

- The packet is sent to its default gateway, R1. This is where NAT happens: R1 translates the source IP address from a private IP to a public IP — typically the router's external interface address.

- This is called source NAT because it translates the source IP address. It's only one option — there are other types of NAT.

# 4. Key NAT Terminology

These four terms matter for every type of NAT, not just static NAT:

| Term | Meaning |
| --- | --- |
| Inside local | The IP address of the inside host, from the perspective of the local network — usually a private address, the one actually configured on the host. |
| Inside global | The IP address of the inside host, from the perspective of the outside network — the address it becomes after NAT, usually a public address. |
| Outside local | The IP address of the outside host, from the perspective of the local network. |
| Outside global | The IP address of the outside host, from the perspective of the outside network. |

# 5. Static NAT

Static NAT = a permanent one-to-one mapping between a private IP and a public IP.

- Global configuration mode — you create the NAT rule/mapping.

- Interface configuration mode — you tell the router which interface is inside and which is outside.

- The router performs the translation when matching traffic passes through.

Interfaces tell the router WHERE; the NAT rule tells it WHAT to translate; the router does the JOB.

## 5.1 How Static NAT Works — the Flow

- PC1 and PC2 send traffic — each packet goes first to their default gateway (the router's inside interface).

- Router receives the packet and checks the NAT table.

- Translation happens: PC1's private IP (192.168.1.10) is translated to its mapped public IP (203.0.113.5); PC2's private IP (192.168.1.11) is translated to its mapped public IP (203.0.113.6).

- Router forwards the packet out through its external interface (the one connected to the internet).

- Return traffic from the internet arrives at the router's external interface, and NAT translates it back to the correct private IP before delivering it to PC1 or PC2.

## 5.2 Analogy: the Post Office

Think of the router as a post office:

- PC1 and PC2 drop letters (packets) at the post office (default gateway).

- The post office replaces their local nicknames (private IPs) with their official street addresses (public IPs).

- Letters go out to the world with those official addresses.

- Replies come back to the post office, which knows exactly which nickname belongs to which address, and delivers them correctly.

Traffic always passes through the default gateway first. NAT translation happens at the router's external interface. Each private IP is mapped to its own public IP in static NAT.

## 5.3 Worked Example: Two PCs, Two Mappings

Suppose two PCs in a local network both want to reach 8.8.8.8. Static NAT is one-to-one, so each PC needs its own mapping:

- R1 maps PC1's private IP 192.168.0.167 to public IP 100.0.0.1.

- Since static NAT can't map two hosts to the same public IP, PC2's private IP 192.168.0.168 is mapped to a different public IP, 100.0.0.2.

- 192.168.0.168 is PC2's inside local address — the actual IP configured on PC2.

- 100.0.0.2 is PC2's inside global address — PC2's IP after NAT.

## 5.4 Limitation of Static NAT

Static NAT allows devices with private IP addresses to communicate over the internet. However, because it requires a strict one-to-one IP mapping, it doesn't really help preserve public IP addresses — if every internal device needs its own public IP anyway, you might as well just configure a public IP on the device directly. Static NAT still has valid use cases, but for address conservation it isn't the most useful kind of NAT.

# 6. Configuring Static NAT

## 6.1 Basic Configuration Steps

- Identify interfaces — mark the inside interface (LAN side) and the outside interface (internet side).

- Create static NAT mappings using:

ip nat inside source static [private_IP] [public_IP]

- Verify NAT operation — use ping or show ip nat translations to confirm.

## 6.2 Example Topology

| Device | Address |
| --- | --- |
| PC1 (LAN) | 192.168.1.10 |
| PC2 (LAN) | 192.168.1.11 |
| Router inside interface (LAN side) | 192.168.1.1 |
| Router outside interface (Internet side) | 203.0.113.1 |
| Public IP mapped to PC1 | 203.0.113.5 |
| Public IP mapped to PC2 | 203.0.113.6 |

## 6.3 Cisco IOS Commands

Router> enable

Router# configure terminal

! Step 1: Define inside and outside interfaces

Router(config)# interface FastEthernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# ip nat inside

Router(config-if)# no shutdown

Router(config)# interface Serial0/0

Router(config-if)# ip address 203.0.113.1 255.255.255.0

Router(config-if)# ip nat outside

Router(config-if)# no shutdown

! Step 2: Create static NAT mappings

Router(config)# ip nat inside source static 192.168.1.10 203.0.113.5

Router(config)# ip nat inside source static 192.168.1.11 203.0.113.6

## 6.4 Verification

- From PC1, ping an internet host — the source should appear as 203.0.113.5.

- From PC2, ping an internet host — the source should appear as 203.0.113.6.

- On the router:

Router# show ip nat translations

→ You'll see the static mappings listed.

## 6.5 Simplified View

- Inside IPs: 192.168.1.10, 192.168.1.11

- Mapped public IPs: 203.0.113.5, 203.0.113.6

- The router performs the translation at the outside interface.

# 7. Reading the NAT Translation Table

When static NAT is used, its entries display permanently in the translation table. The table lists, in order: inside global address first, then inside local address.

- Pro — the protocol used for that translation.

- Inside global column — next to each IP address is a port number.

- Inside local column — even though the IP address has been translated, the router does not translate the port number under static NAT; it stays the same.

Two further terms appear in the table:

- Outside local — the IP address of the outside host, from the perspective of the local network.

- Outside global — the IP address of the outside host, from the perspective of the outside network.

# 8. Managing NAT Translations

## 8.1 Clearing Translations

clear ip nat translations removes current NAT translation entries from the router's NAT table.

- NAT translations are the temporary records the router keeps of which private IP maps to which public IP (and ports, if PAT is used).

- Clearing them forces the router to rebuild the table when new traffic flows.

Router# clear ip nat translations *

! deletes all NAT entries

Router# clear ip nat translation tcp 203.0.113.5 80 192.168.1.10 1025

! deletes a single TCP translation entry

Why use it:

- Troubleshooting — resets the table if NAT isn't working correctly.

- Testing — clear old entries so new configuration rules apply immediately.

- Freeing resources — removes stale or unused mappings.

Clearing translations does not remove the NAT configuration itself — only the active mappings. As soon as devices send traffic again, new translations are created automatically. Static NAT mappings remain in the table permanently and are unaffected by this command.

Analogy: NAT translations are like a guest list at a party. clear ip nat translations erases the guest list — but the rules of who's allowed in (the static NAT config) are still there, so the list refills as soon as guests arrive again.

## 8.2 Viewing Static Rules

show ip nat static displays the static NAT mappings you configured — the permanent rules set with ip nat inside source static — regardless of whether traffic is currently flowing. This is different from show ip nat translations, which shows active sessions (dynamic NAT, PAT, and static entries in use).

Router(config)# ip nat inside source static 192.168.1.10 203.0.113.5

Router(config)# ip nat inside source static 192.168.1.11 203.0.113.6

Router# show ip nat static

Static mappings:

192.168.1.10 -> 203.0.113.5

192.168.1.11 -> 203.0.113.6

Analogy: static NAT rules are like permanent entries in a notebook, while the translations table is the current "guest list" of who's actually inside the party. show ip nat static is checking the notebook; show ip nat translations is checking the guest list.

## 8.3 Command Reference

| Command | Shows |
| --- | --- |
| show ip nat translations | All active NAT sessions — dynamic, PAT, and static entries currently in use |
| show ip nat static | Only the configured static NAT rules, whether or not traffic is currently flowing |
| clear ip nat translations * | Clears all dynamic translation entries (static mappings are unaffected) |
| clear ip nat translation tcp <pub-ip> <port> <priv-ip> <port> | Clears one specific translation entry |

# 9. Key Takeaways

- NAT translates the source and/or destination IP address of a packet — most commonly so private hosts can reach the internet.

- Three short-term fixes for IPv4 exhaustion: CIDR, private addressing, and NAT.

- Private ranges (RFC 1918): 10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16.

- Four key terms: inside local, inside global, outside local, outside global.

- Static NAT = permanent one-to-one private-to-public IP mapping; simple but doesn't conserve public addresses.

- Core commands: ip nat inside / ip nat outside on interfaces, then ip nat inside source static <private_IP> <public_IP>.

- show ip nat translations = active sessions; show ip nat static = configured static rules; clear ip nat translations = wipes dynamic entries only.