STP Part 2: Port States and Timers

CCNA study notes · Spanning Tree Protocol

# Part 2 outline

| Topic | Status in these notes |
| --- | --- |
| STP port states / timers | Covered below |
| STP BPDU | To cover |
| STP optional features | To cover |
| STP configuration | To cover |

# 1. STP port states

| Blocking Stable | → | Listening Transitional · 15 s | → | Learning Transitional · 15 s | → | Forwarding Stable |
| --- | --- | --- | --- | --- | --- | --- |

| STP port state | Stable / transitional | Who ends up here |
| --- | --- | --- |
| Blocking | Stable | Non-designated ports |
| Listening | Transitional | Root and designated ports, on the way up |
| Learning | Transitional | Root and designated ports, on the way up |
| Forwarding | Stable | Root and designated ports |

- Root and designated ports remain in the stable forwarding state.

- Non-designated ports remain in the stable blocking state.

- Listening and learning are transitional states. A port passes through them when an interface is activated, or when it moves to forwarding because of a topology change.

Memory trick B → L → L → F: Blocking → Listening → Learning → Forwarding Block = wait and check the topology. Listen = STP decides the roles. Learn = build the MAC table. Forward = work normally.

# 2. What each state does

## 1) Blocking

- Non-designated ports are in the blocking state.

- Effectively disabled to prevent loops.

- Does not send or receive regular network traffic.

- Receives STP BPDUs, but does not forward them.

- Does not learn MAC addresses.

## 2) Listening

- Only designated or root ports enter the listening state.

- Lasts 15 seconds by default, set by the forward delay timer.

- Only sends and receives STP BPDUs. The switch uses this time to work out the root bridge and the port roles.

- Does not send or receive regular traffic.

- Does not learn MAC addresses from regular traffic arriving on the interface.

## 3) Learning

- After listening, a designated or root port enters the learning state.

- Also lasts 15 seconds by default. The same forward delay timer is used for both listening and learning, so by default it takes 30 seconds in total to pass through both states and reach forwarding.

- Only sends and receives STP BPDUs. No regular traffic yet.

- Main difference from listening: the interface now learns MAC addresses from regular traffic arriving on it.

## 4) Forwarding

- Root and designated ports are in the forwarding state.

- The port operates as normal: sends and receives BPDUs, sends normal traffic, and learns MAC addresses.

# 3. Comparison table

| State | Type | Sends/receives BPDUs | Learns MAC addresses | Forwards user traffic | Duration |
| --- | --- | --- | --- | --- | --- |
| Blocking | Stable | Receives only | No | No | Until topology changes |
| Listening | Transitional | Yes | No | No | 15 s |
| Learning | Transitional | Yes | Yes | No | 15 s |
| Forwarding | Stable | Yes | Yes | Yes | Until topology changes |

# 4. Timers (defaults)

| Timer | Default | Purpose |
| --- | --- | --- |
| Hello | 2 s | How often BPDUs are sent. |
| Forward delay | 15 s | Time spent in listening and again in learning (30 s in total). |
| Max Age | 20 s | How long a switch keeps a BPDU before treating it as expired. |

Common mix-up Do not memorize “blocking = 20 seconds, then listening” as a direct sequence. The 20 s is the Max Age timer. The normal transition timers are 15 s listening + 15 s learning.

# 5. How a port decides: a three-switch example

SW1, SW2 and SW3 are connected in a triangle. Each switch does not pick its own root. They exchange BPDUs and all agree on the same root bridge for the VLAN. There is one STP calculation per VLAN.

| Switch | Priority | MAC | Result |
| --- | --- | --- | --- |
| SW1 | 32768 | AAAA | Root bridge (priorities tie, lowest MAC wins) |
| SW2 | 32768 | BBBB |  |
| SW3 | 32768 | CCCC |  |

- Root bridge: SW1 wins, so all its active ports are designated ports.

- Root ports: SW2 and SW3 each pick the port with the best path toward SW1.

- SW2–SW3 link: STP compares paths to the root bridge. One side becomes a designated port and the other side becomes non-designated (blocking). This breaks the loop.

- The ports that end up as root or designated then go Listening → Learning → Forwarding. The blocked port stays in blocking.

The physical triangle still exists, but STP logically builds a loop-free tree with SW1 at the top. That is why it is called Spanning Tree Protocol.

# 6. Q&A / clarifications

What is the difference between listening and learning?

Listening: STP decides the topology and the port roles, and no MAC addresses are learned. Learning: the port starts learning source MAC addresses, but still does not forward user traffic.

Does the learning state check whether the neighbor is a switch or a PC?

No. BPDUs already tell a switch that its neighbor is a switch. In the learning state the switch learns the source MAC address of frames arriving on the port.

What is the purpose of learning MAC addresses in the learning state?

To build the MAC address table (MAC address → port) before the port starts forwarding. Then the switch already knows where each device is and sends frames only out the right port instead of flooding them.

| MAC address | Port |
| --- | --- |
| AAAA (PC1) | Fa0/1 |
| BBBB (PC2) | Fa0/2 |
| CCCC (PC3) | Fa0/3 |

A frame from PC1 to BBBB goes out Fa0/2 only. PC3 does not receive it.

Why does a port take time to turn from orange to green?

With traditional STP the port goes through blocking, listening and learning before forwarding. That is why there is a delay when you plug in a PC.

CCNA tip PortFast on ports facing end devices (PCs) lets them reach forwarding almost immediately. Rapid PVST+ converges much faster than traditional STP and is common in modern Cisco networks.

# 7. Still to cover

- STP BPDU (fields and format)

- STP optional features

- STP configuration commands