# Subnetting Notes

A summary of core subnetting concepts: CIDR, address classes, special prefixes, subnet/host calculations, and Fixed-Length Subnetting (FLSM) vs. VLSM.

# CIDR (Classless Inter-Domain Routing)

An IPv4 address is 32 bits, split into four octets (groups of 8 bits).

| Class | Prefix | First Octet (binary) | First Octet Range (decimal) |
| --- | --- | --- | --- |
| A | /8 | 0xxxxxxx | 0 – 127 |
| B | /16 | 10xxxxxx | 128 – 191 |
| C | /24 | 110xxxxx | 192 – 223 |
| D | — | 1110xxxx | 224 – 239 |
| E | — | 1111xxxx | 240 – 255 |

Example (Class B): the first two bits are fixed, leaving 14 bits for the network portion → 2¹⁴ = 16,384 possible networks, and 16 bits for hosts.

Who assigns address blocks? The IANA (Internet Assigned Numbers Authority), a non-profit, assigns IPv4 networks to companies/organizations based on their size. Large companies may get a Class A or B network; smaller companies typically get a Class C network.

# Special Prefix Lengths

- /30 — 2 host bits → 4 total addresses (2 usable). The other 2 addresses are the network address and broadcast address, so on a point-to-point link (e.g., R1–R2) 2 addresses go unused by hosts.

- /31 — mask 255.255.255.254. Only 1 host bit, so by the standard formula (2¹ − 2 = 0) there are 0 usable addresses. Normally unusable, but it is a special-case exception used specifically for point-to-point links, where the usual rule is broken.

- /32 — mask 255.255.255.255. The entire address is the network portion (host formula gives -1, i.e. it doesn't apply). Never used to configure a network or interface — instead it's used to create a static route to one specific host.

# Subnetting Formulas

- Number of subnets = 2ⁿ, where n = number of borrowed bits

- Number of usable hosts = 2ⁿ − 2, where n = number of remaining host bits

When subnetting, always ask: does the number of hosts I need allow me to divide this network the way I want, and what class is the original network in?

# How to Identify a Subnet

## Example 1: What subnet does host 192.168.5.57/27 belong to?

- Convert to binary: 11000000.10101000.00000101.00111001

- /27 means 3 bits are borrowed from the last octet. Zero out the remaining host bits: 11000000.10101000.00000101.00100000 → 192.168.5.32

- Answer: the host belongs to subnet 192.168.5.32

## Example 2: What subnet does host 192.168.29.219/29 belong to?

- Convert to binary: 11000000.10101000.00011101.11011011

- Apply the same method as Example 1 (zero out the host bits based on the /29 mask) to find the subnet.

# Fixed-Length Subnetting (FLSM) vs. VLSM

When you subnet a network, you can split it up in two different ways.

## Fixed-Length Subnetting (FLSM)

- Every subnet created from the network is the same size — you borrow a fixed number of bits once and apply it everywhere.

- Simple to plan and calculate, but often wastes addresses: a point-to-point link that only needs 2 usable addresses still gets the same size subnet as a LAN that needs 50 hosts.

- Good for networks where all subnets genuinely need a similar number of hosts.

Example: Given 192.168.1.0/24 and a need for 4 subnets, you borrow 2 bits to make four equally-sized /26 subnets (each with 62 usable hosts) — even if some of those subnets only need a handful of addresses.

## VLSM (Variable-Length Subnet Masking)

- Lets you subnet a network into different-sized subnets, each sized to match how many hosts it actually needs.

- Far more efficient — little to no address waste — but requires more careful planning, since each subnet's range must be calculated in sequence and must not overlap with the others.

- The standard real-world approach for enterprise/ISP networks, and for point-to-point links (often sized as /30 or /31).

How to design with VLSM:

- List all required subnets and sort them from largest host requirement to smallest. Always allocate the biggest subnet first.

- For each subnet, determine the smallest prefix length that satisfies 2ⁿ − 2 ≥ required hosts (n = host bits).

- Assign that subnet the next available block of addresses from the pool, then move the "next available address" pointer past it.

- Repeat for the next-largest requirement, continuing until all subnets are assigned.

Example: Given 192.168.1.0/24 with three needed subnets — 100 hosts, 50 hosts, and a point-to-point link (2 hosts):

| Subnet | Hosts Needed | Prefix | Usable Hosts | Address Range |
| --- | --- | --- | --- | --- |
| A | 100 | /25 | 126 | 192.168.1.0 – 192.168.1.127 |
| B | 50 | /26 | 62 | 192.168.1.128 – 192.168.1.191 |
| C | 2 (P2P) | /30 | 2 | 192.168.1.192 – 192.168.1.195 |

Notice the largest subnet is allocated first (starting at the beginning of the block), and each subsequent subnet starts right after the previous one ends — this is what keeps VLSM address-efficient and non-overlapping.

## Quick Comparison

|  | Fixed-Length (FLSM) | VLSM |
| --- | --- | --- |
| Subnet sizes | All equal | Vary per need |
| Address efficiency | Lower (wastes addresses) | Higher (minimal waste) |
| Planning complexity | Simple | Requires careful sequential allocation |
| Typical use | Simple/uniform networks | Enterprise networks, WAN links, real-world designs |

Converted from personal study notes — CIDR, prefix lengths, subnet identification, and FLSM vs. VLSM.